# SIMATS ENGINEERING

ASSESSMENT TOOL 1: Analytical Problem Solving

COURSE NAME:COMPUTER VISION

CODE:ITA0510

CO2: Apply image enhancement and segmentation techniques for extracting meaningful regions from images. (BTL 3)

Assessment Tool Weightage: 50%

## QUESTIONS: 5	Total Marks50	Duration :60 Minutes Annexure A

Analytical Problem Solving

### Code of Conduct:

I  S .Varsha Reddy(192572177)  certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

### Signature of the student:S.Varsha Reddy

# RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) | Marks (100) |
| --- | --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |  |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |  |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |  |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |  |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |  |

## Grey Level Transformation

  - Explain the purpose of grey level transformation in image enhancement.

  - Apply a negative transformation s=255-r to the pixel values: r=[50,100,150,200]

Compute the transformed values.

Ans: (a) Purpose of Grey Level Transformation in Image Enhancement

Grey level transformation is a point processing technique used to modify the intensity values of pixels in an image. It improves the visual quality of an image by changing the brightness and contrast of individual pixels.

Purpose:

- Enhances the visibility of image details.

- Improves image contrast and brightness.

- Highlights important features in an image.

- Suppresses unwanted details or noise.

- Serves as a preprocessing step for image analysis and computer vision applications.

(b) Negative Transformation

Given:

Negative transformation formula:

s=255-r

Where:

- r= Original pixel value

- s= Transformed pixel value

- Maximum grey level = 255 (8-bit image)

Original pixel values:

r=50 100  150  200
Solution Process

Using

s=255-r

Step 1: Calculate for r=50

s=255-50=205

Step 2: Calculate for r=100

s=255-100=155

Step 3: Calculate for r=150

s=255-150=105

Step 4: Calculate for r=200

s=255-200=55
Final Answer

s=205  155  105  55

Interpretation of Results

The negative transformation reverses the intensity values of the image. Pixels with lower intensity become brighter, while pixels with higher intensity become darker. This transformation is particularly useful for enhancing details in images where dark regions contain important information, such as medical images (X-rays) and satellite images. The transformed pixel values remain within the valid 8-bit grey level range (0–255).

## Log Transformation

  - Explain how log transformation enhances low-intensity pixels.

  - Given s=clog(1+r), where c=10, compute s for: r=[0,10,50,100]

Ans:

(a)Log transformation is an image enhancement technique that expands the values of low-intensity (dark) pixels while compressing the values of high-intensity (bright) pixels. This makes details in dark regions more visible without significantly affecting bright areas.

Purpose:

- Enhances dark regions of an image.

- Improves the visibility of low-intensity details.

- Compresses high-intensity pixel values to prevent excessive brightness.

- Widely used in medical imaging, satellite imaging, and image preprocessing.

- Helps improve the dynamic range of an image.

(b) Compute the Log Transformation

Given:

Log transformation formula:

s=clog⁡1+r

Where:

- c=10

- r= Original pixel value

- log= Common logarithm (base 10)

Original pixel values:

r=[0,10,50,100]

Using the formula:

s=10log⁡1+r

Step 1: For r=0

s=10log⁡1+0=10log⁡1=100=0

Step 2: For r=10

s=10log⁡11
log⁡11=1.0414
s=101.0414=10.41

Step 3: For r=50

s=10log⁡51
log⁡51=1.7076
s=101.7076=17.08

Step 4: For r=100

s=10log⁡101
log⁡101=2.0043
s=102.0043=20.04
Final Answer

s=0.00  10.41  17.08  20.04

Interpretation of Results

The log transformation increases the intensity of low-value pixels more than high-value pixels. As a result, dark areas of the image become brighter and reveal more details, while bright regions are compressed to avoid over-enhancement. This technique is especially useful in medical imaging, satellite image processing, and computer vision, where important information is often hidden in low-intensity regions.

## Histogram Processing

  - Explain the role of histogram equalization in contrast enhancement.

  - Given a 3-bit image with histogram:

| Intensit y | Frequenc y |
| --- | --- |
| 0 | 2 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |

Compute the cumulative distribution and equalized intensity values.

Ans:

(a)

Histogram equalization is an image enhancement technique that redistributes the intensity values of an image so that they are spread more uniformly over the available grey-level range. This improves the overall contrast, especially in images where the pixel values are concentrated within a narrow intensity range.

Purpose:

- Enhances image contrast.

- Improves the visibility of details in dark and bright regions.

- Spreads pixel intensities across the full grey-level range.

- Produces a more uniform histogram.

- Commonly used in medical imaging, satellite imaging, and computer vision applications.

A 3-bit image has:

- Number of grey levels:

L=23=8

Intensity levels range from 0 to 7.

Given histogram:

| Intensity (r) | Frequency (f) |
| --- | --- |
| 0 | 2 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |

Total number of pixels:

- N=2+2+4+8=16

Histogram equalization is performed using the formula:

sk=L-1×CDFrk

Where:

- L=8

- L-1=7

- CDF= Cumulative Distribution Function

The resulting value is rounded to the nearest integer.

Solution Process

Step 1: Calculate Probability

Pr=fN

| Intensity | Frequency | Probability Pr |
| --- | --- | --- |
| 0 | 2 | 2/16 = 0.125 |
| 1 | 2 | 2/16 = 0.125 |
| 2 | 4 | 4/16 = 0.250 |
| 3 | 8 | 8/16 = 0.500 |

Step 2: Calculate Cumulative Distribution Function (CDF)

| Intensity | Probability | CDF |
| --- | --- | --- |
| 0 | 0.125 | 0.125 |
| 1 | 0.125 | 0.250 |
| 2 | 0.250 | 0.500 |
| 3 | 0.500 | 1.000 |

Step 3: Calculate Equalized Intensity

Using

s=7×CDF

| Intensity (r) | CDF | 7×CDF | Equalized Intensity (s) |
| --- | --- | --- | --- |
| 0 | 0.125 | 0.875 | 1 |
| 1 | 0.250 | 1.750 | 2 |
| 2 | 0.500 | 3.500 | 4 |
| 3 | 1.000 | 7.000 | 7 |

Final Answer

Cumulative Distribution Function (CDF)

| Intensity | CDF |
| --- | --- |
| 0 | 0.125 |
| 1 | 0.250 |
| 2 | 0.500 |
| 3 | 1.000 |

Equalized Intensity Values

| Original Intensity | Equalized Intensity |
| --- | --- |
| 0 | 1 |
| 1 | 2 |
| 2 | 4 |
| 3 | 7 |

Interpretation of Results

Histogram equalization redistributes the pixel intensities over the available grey-level range (0–7), resulting in improved image contrast. The original intensity values 0, 1, 2, and 3 are mapped to 1, 2, 4, and 7, respectively. This wider distribution enhances image details and makes objects more distinguishable, which is particularly beneficial in medical imaging, satellite image analysis, and computer vision.

## Spatial Filtering (Smoothing)

  - Explain the purpose of spatial filtering for smoothing.

  - Apply a 3×3 averaging filter to the following region: 10 20 30 20 30 40 30 40 50

Compute the new center pixel value.

Ans: (a) Spatial filtering (smoothing) is an image enhancement technique used to reduce noise and small intensity variations by replacing each pixel value with the average of its neighboring pixels. It produces a smoother image and improves image quality.

Purpose:

- Reduces image noise.

- Smooths unwanted intensity variations.

- Removes small details and irregularities.

- Produces a cleaner image for further processing.

- Used as a preprocessing step in computer vision and image analysis.

b) Apply a 3 × 3 Averaging Filter

Given Region

102030203040304050

Method

For a 3 × 3 averaging filter, the new center pixel is calculated as:

New Center Pixel=Sum of all 9 pixels9

Step 1: Find the Sum

10+20+30+20+30+40+30+40+50=270

Step 2: Divide by 9

2709=30

Final Answer

New Center Pixel Value=30

Interpretation of Results

The averaging filter replaces the center pixel with the average value of its neighboring pixels. This smooths the image, reduces noise, and preserves the overall brightness. The new center pixel value is 30, making the image less noisy and more suitable for further image processing tasks.

## Spatial Filtering (Sharpening)

  - Explain how sharpening enhances image details.

  - Apply the Laplacian mask:

[0 -1	0}, {-1	4 -1], { 0 -1	0 ]

to the matrix:

10 10 10, 10 20 10, 10 10 10

Compute the output at the center.

Ans: (a) Sharpening is an image enhancement technique used to highlight edges and fine details by emphasizing sudden changes in pixel intensity. It improves the clarity and sharpness of an image, making object boundaries more distinct.

Purpose:

- Enhances edges and fine details.

- Increases image sharpness.

- Improves object boundaries.

- Makes important features more visible.

- Used in medical imaging, satellite imaging, and computer vision.

(b) Apply the Laplacian Mask

Given Laplacian Mask

0-10-14-10-10

Given Image Matrix

101010102010101010

Simple Calculation

0×10+-1×10+0×10
+-1×10+4×20+-1×10
+0×10+-1×10+0×10
=0-10+0-10+80-10+0-10+0
=80-40
=40

Final Answer

Output at the Center=40

Interpretation of Results

The Laplacian filter highlights the intensity difference between the center pixel and its neighboring pixels. Since the center pixel (20) is brighter than all surrounding pixels (10), the output value (40) indicates a strong edge or detail at the center. This enhances the sharpness of the image and makes edges more prominent.